Ohsung Steel ERP 2.0 · Production Floor Tools

Leveling Production Report

Thursday, September 3, 2026 - Based on leveling-line PLC telemetry (LEVELING_DATA) - How equipment settings, box handling, and uncoiler tension behave as each coil's length progresses

# 1. Overview

Two leveling jobs were completed on the leveling line on Thursday, September 3, 2026. The table below combines the GreenERP job order (erp_data) with leveling-line PLC telemetry (LEVELING_DATA, MariaDB osungsteel.servehttp.com). Total length and sheet count are not read from a direct counter - they are derived by detecting the moment the in-stroke position value (MOVELENSV) resets back toward zero after each sheet, then accumulating that sheet's cut-length setting (CUTLENSET). See Section 7 for methodology and caveats.

| Coil ID | Customer | Spec | Weight (kg) | Work window (KST) | Est. total length | Est. sheets |
| --- | --- | --- | --- | --- | --- | --- |
| EPD2871C01 | Daehan Gangjae ((주)대한강재) | 0.76X4XC | 10,410 | 08:34-10:27 (1h 53m) | 863.4 m | 289 |
| 59A2078B | Yuchang ((주)유창) | 1.15X1200XC | 10,700 | 11:37-15:30 (3h 53m) | 992.4 m | 332 |

Both jobs used essentially the same cut length per sheet (~2.99 m) and both ran well inside the normal 08:30-17:30 work day. Coil 1 started shortly after the shift began (08:34) and finished at 10:27; after a changeover gap of about 1h 10m (10:27-11:37), Coil 2 ran from 11:37 to 15:30. The line finished both jobs by mid-afternoon, roughly two hours before the shift's normal close.

# 2. Job 1 - EPD2871C01 (Daehan Gangjae, 0.76mm)

## 2.1 Equipment settings (fixed for the entire length)

Settings were finalized once at 08:48:02 - about 14 minutes after the PLC log for this job begins at 08:34:03 - and never changed again through the 10:27:06 finish (a 1mm wobble in CUTLENSET between 2994 and 2995mm at 09:01-09:03 is sensor rounding noise, not a real change).

| Setting | Value | Notes |
| --- | --- | --- |
| CUTLENSET (cut-length setting) | 2,994 mm (~2.994 m) | Fixed from 08:48:02 to end |
| SPEEDSET (speed setting) | 25 | Fixed for the entire run, never changed |
| ACCTM (acceleration time) | 10.0 sec | Fixed |
| DECTM (deceleration time) | 1.0 sec | Fixed |
| SLOWLENSET (slow-down length setting) | 50 mm | Fixed |
| INVERTERSPEEDCOEFF (inverter speed coefficient) | 23.7 - 24.0 | Measurement-noise level only, effectively fixed |

## 2.2 Measured performance vs. coil length

Figure 1. EPD2871C01 - inverter load (%) and cycle time per stroke (sec) vs. cumulative coil length (m)

- Warm-up phase (0-220m, roughly the first 25% of the coil): load climbs 6.5% -> 22.4% -> 36.9% -> ~48%, while cycle time shortens from 9.06 sec to ~4.8 sec. Settings did not change at all in this phase, so this is a real warm-up in the machine's realized performance, not a settings adjustment.

- Steady-state phases (220-432m and 504-863m): load holds at 47-51%, cycle time at 4.8-5.1 sec.

- Low-load dip (431.7-503.65m, 09:39-10:07 KST, ~28 minutes): load falls to as low as 9.5% and cycle time returns to a warm-up-like 9.04 sec. Since no setting changed during this window, this looks like a temporary stop or slow-down on the floor (e.g. checking material) rather than a control-parameter change.

- Box handling: per LEVELING_DATA's BOXIDX field, this entire coil was processed as a single box - there was no box change during this run.

- Because this coil has only one box, the 28-minute dip above is NOT explained by a box change (unlike Coil 2's pattern in Section 3). Its cause is not visible in the PLC settings and would need to be confirmed with the floor team - e.g. a splice/material check or an unlogged stoppage.

# 2b. Uncoiler Tension (PR-DTC-3100)

The uncoiler ahead of the leveling line has its own legacy tension-controller display (no data port), read by camera + OCR (Groq/Qwen vision) into the ai_tension_readings table roughly every 5 minutes while the monitoring script is running. A reading has no coil ID or length of its own, so it is attributed here by two independent signals: (a) time overlap with each coil's LEVELING_DATA run window, and (b) an exact match between the reading's OCR'd thickness_um field and each coil's spec (760um = 0.76mm for Coil 1, 1150um = 1.15mm for Coil 2).

With the timeline corrected, the two signals agree: 63 of that day's 64 readings carry exactly one of those two thickness values, and each group's time range sits right on top of the matching coil's run window. So - unlike this report's earlier draft - there is good overlapping tension telemetry for both of today's jobs.

Figure 1b. Uncoiler tension output % vs. time of day, split by matched coil (thickness_um)

| Field | Coil 1 (760um / 0.76mm) | Coil 2 (1150um / 1.15mm) |
| --- | --- | --- |
| Reading window (KST) | 09:01:20 - 11:32:34 | 11:56:52 - 15:40:29 |
| Readings | 31 | 32 |
| Avg. output % (=tension_set) | 69.8 % | 70.7 % |
| Tension steps observed | 90.0 -> 70.9 -> 60.0 -> 50.5 -> 40.1 | 90.0 -> 60.3 -> 50.0 -> 40.0 -> 20.1 |
| Coverage vs. PLC run window | Starts ~27 min after PLC start (08:34); continues ~65 min past PLC finish (10:27) into the changeover | First reading at start, then a monitoring gap (below); resumes mid-run; last reading ~10 min after PLC finish (15:30) |

- Taper-tension pattern: both coils show the same shape - tension holds at a high plateau (90%) for most of the run, then steps down through 4-5 discrete levels as the run nears completion, rather than drifting continuously. This matches the PR-DTC-3100 controller's designed purpose - reducing tension as the remaining coil diameter shrinks, to avoid core crush/telescoping - and the fact that both coils independently show the same 5-step pattern is a good sign the behavior is a real, repeatable control response rather than noise.

- Coil 1's step-down continues into the post-job changeover (60.0% at 10:32, still 40.1% at 11:32, one reading before Coil 2's tension appears) - consistent with the old coil physically remaining on the uncoiler stand, still unwinding down to its core, until it is swapped out for Coil 2.

- Monitoring gap during Coil 2: there is a ~67-minute gap in readings from 11:56:52 to 13:04:15, which falls entirely inside Box 1 (11:37-13:37, Section 3b). This is a data-collection gap, not a production stoppage - LEVELING_DATA shows the line running normally through this window. Worth checking whether the OCR monitor script paused or restarted around this time.

- Both reading windows start after and/or end slightly after the matching PLC run window's edges (Coil 1: readings start 27 min into the run; Coil 2: last reading is 10 min after the run finished) - a small, expected lag from the ~5-minute OCR polling interval plus the monitor continuing to read the display briefly after a job ends.

# 3. Job 2 - 59A2078B (Yuchang, 1.15mm)

## 3.1 Equipment settings

The previous job's (Coil 1, 0.76mm) SPEEDSET=25 was still in effect when Coil 2 began, then was corrected to 15 at the 3m mark (11:55:58) - consistent with Yuchang's thicker spec (1.15mm) calling for a lower speed. It did not change again for the rest of the 992m run. CUTLENSET was also nudged from 2994 to 2993.5mm at the 15m mark, then held fixed.

| Setting | Value | Notes |
| --- | --- | --- |
| CUTLENSET (cut-length setting) | 2,993.5 mm (~2.994 m) | Fixed after the 15m mark |
| SPEEDSET (speed setting) | 15 | Corrected from 25 to 15 at the 3m mark, then fixed (reflects thicker spec vs. Coil 1) |
| ACCTM / DECTM | 10.0 sec / 1.0 sec | Fixed |
| SLOWLENSET (slow-down length setting) | 50 mm | Fixed |
| INVERTERSPEEDCOEFF (inverter speed coefficient) | 24.2 - 24.5 | Fixed |

## 3.2 Measured performance vs. coil length

Figure 2. 59A2078B - inverter load (%) and cycle time per stroke (sec) vs. cumulative coil length (m)

- Unlike Coil 1's single warm-up curve, this coil shows a repeating sawtooth: low-load segments (5-14%, cycle time 8.7-12.7 sec) alternate with high-load segments (24-31%, cycle time 7.0-7.5 sec) about every 80-90m, 6+ times over the run.

- Settings never changed again after the 3m mark - so the repeated rise and fall in load is a real, measured effect, not the operator re-tuning the machine.

# 3b. Box breakdown - Coil 2

Coil 2 was processed as 4 separate boxes (BOXIDX 1-4). The table and chart below show each box's production time and the gap between boxes (i.e. the box-change pause).

| Box | Start (KST) | End (KST) | Duration | Gap before next box | Avg. load % |
| --- | --- | --- | --- | --- | --- |
| 1 | 11:36:58 | 13:37:06 | 2h 00m | - | 7.7 % |
|  |  |  |  | 11m 58s (box change) |  |
| 2 | 13:49:04 | 14:28:44 | 39m 40s |  | 17.0 % |
|  |  |  |  | 4m 36s (box change) |  |
| 3 | 14:33:20 | 15:07:17 | 33m 57s |  | 19.2 % |
|  |  |  |  | 2 sec (negligible) |  |
| 4 | 15:07:19 | 15:30:00 | 22m 41s | - | 24.7 % |

Figure 3. Coil 2 59A2078B - box production spans and the gaps between them

- This confirms the operator's own explanation: leveling packs finished sheets into boxes underneath each coil, and the brief pauses seen in the load/cycle-time chart correspond to stopping to swap out a full box. The two clearest gaps (12 minutes after Box 1, ~4.6 minutes after Box 2) line up with the low-load dips visible in Figure 2.

- Box-average load rises from box to box (7.7% -> 17.0% -> 19.2% -> 24.7%), which matches the same warm-up-then-plateau behavior seen in Coil 1, just interrupted by 3 box changes instead of running as one continuous block.

- Note the asymmetry with Coil 1 (Section 2): Coil 1's own 28-minute dip is not a box change, since it was processed as a single box throughout. So box changes explain Coil 2's repeating pattern well, but do not explain Coil 1's one dip - that one still needs a floor-side explanation.

# 4. Comparing the two jobs

Figure 4. Load (%) vs. progress through the coil (%) - both jobs

- Common pattern: both jobs show the same operating model - settings are locked in once at the start and held for the whole run, while only the measured values (load %, cycle time) move with length. The leveling line is not being re-tuned in real time as a coil progresses; it is set once to match the job spec and then left alone.

- Thickness vs. SPEEDSET: the 0.76mm coil ran at SPEEDSET 25, the thicker 1.15mm coil at SPEEDSET 15 - thicker material set to run slower (only 2 data points here, so this should be confirmed against more coils before treating it as a rule).

- Difference: Coil 1 only shows a warm-up in its first ~25% of length and is stable for the remaining 75%; Coil 2 never reaches a single stable plateau, instead repeating the rise-and-fall pattern tied to its 4 box changes. Coil 1 being a single box vs. Coil 2's four boxes is the most likely reason for that difference.

# 5. Key insights

- (1) Leveling-line settings (CUTLENSET, SPEEDSET, ACCTM, DECTM, SLOWLENSET, INVERTERSPEEDCOEFF) are not adjusted gradually as a coil's length progresses - they are locked in once at job start to match the spec, then held fixed to the end. Operators are not re-tuning the line in real time during a run.

- (2) What does change with length is measured performance (load % / cycle time), and that change is explained almost entirely by two things: (a) machine warm-up early in a run, and (b) box-change pauses - confirmed directly for Coil 2 (Section 3b), and matching the pattern seen in Coil 1's own warm-up, even though Coil 1's mid-run dip is not itself a box change.

- (3) Finding (1) supports the case for the already-planned 'Leveler AI Helper / length-based control guide' (planned 2026-09-03, architecture.md Section 11): since operators are not currently adjusting settings by length, an automated length-based recommendation tool would add real, new value rather than just reproducing what operators already do.

- (4) Sheet-count method check: this report's MOVELENSV-reset method gives 289 sheets for Coil 1 and 332 for Coil 2. The existing leveling-detail screen's method (span-time / average cycle time) gives 861 and 1,359 - about 3.0x and 4.1x too high. This directly reconfirms, with fresh data, the known issue already documented in architecture.md that CYCLETM is an in-stroke counter and over-counts sheets 3-4x.

- (5) Uncoiler tension follows a 'taper' pattern rather than a continuous one: both coils independently show tension holding at a 90% plateau for most of the run, then stepping down through 4-5 discrete levels as each coil nears completion (Section 2b) - direct evidence the PR-DTC-3100 controller is reducing tension as coil diameter shrinks, exactly as it is designed to do. The one gap in this data is a ~67-minute OCR monitoring blackout during Coil 2's Box 1, worth checking against the OCR script's logs.

# 6. Data and methodology

Data sources: the leveler-explore Supabase Edge Function (new leveling_length_profile mode, added 2026-09-04) against LEVELING_DATA (leveling PLC telemetry, MariaDB osungsteel.servehttp.com) and erp_data (job orders); ai_tension_readings (Supabase Postgres) for uncoiler tension.

- Deriving cumulative length: LEVELING_DATA has no column for a coil's total accumulated length - only MOVELENSV, which rises from 0 toward CUTLENSET within each stroke and resets for the next sheet. This analysis flags a reset whenever MOVELENSV drops more than 100mm from the previous reading, and accumulates each completed sheet's CUTLENSET to back into a cumulative length.

- Cross-check: Coil 2 (59A2078B, 1.15mm x 1,200mm, 10,700kg) converts to an estimated ~987.7m via steel density (7,850 kg/m3), within 0.5% of this report's PLC-derived 992.4m - supporting the method's reliability. Coil 1's spec string (0.76X4XC) doesn't clearly encode a width, so the same cross-check wasn't possible for it.

- Limitations: cumulative length and sheet counts are estimates, not direct counter readings. Improving accuracy would need (a) a real cut counter tag in the PLC, (b) the OCR/production tooling to record COIL_ID and sheet count at capture time, and (c) enforcing COIL_ID tagging across all runs - all already listed as follow-ups in the existing plan (레벨러_AI헬퍼_개발기획_정리.md).

- Uncoiler tension readings (ai_tension_readings) carry no coil ID or length of their own; each reading is attributed here to a coil using two independent signals - time overlap with that coil's LEVELING_DATA run window, and an exact match between the reading's OCR'd thickness_um and the coil's spec thickness (760um/0.76mm for Coil 1, 1150um/1.15mm for Coil 2). Both signals agree for 63 of the day's 64 readings (Section 2b).

- An earlier version of this report incorrectly added 9 hours to LEVELING_DATA's TIMESTAMP field when converting to KST. LEVELING_DATA.TIMESTAMP is already stored in KST (confirmed by an hour-of-day histogram of the raw values, which peaks exactly at the plant's 08:00-17:40 business hours with no shift needed), while ai_tension_readings.captured_at is genuine UTC and does need the +9h conversion. All clock times in this report have been corrected accordingly.

# 7. Recommendations

- Prioritize the planned length-based control guide (including cycle-rhythm prediction and box-change alerts), since this report confirms its core premise: operators fix settings at job start and do not adjust them by length.

- Add automatic detection of 'measured values drop with no setting change' windows - like Coil 1's 28-minute dip and Coil 2's repeating box-change dips - to the live dashboards, so unusual stops can be traced after the fact.

- Investigate the ~67-minute uncoiler-tension OCR monitoring gap during Coil 2's Box 1 (11:56:52-13:04:15) to see whether it is a script pause/restart, a camera issue, or something else worth fixing so future reports have continuous coverage.

- Work with the floor team to add a real cut-counter PLC tag, to remove the 3-4x sheet-count discrepancy documented in Section 5(4).

Prepared by: Ohsung Steel ERP 2.0 - Automated analysis report - 2026-09-04